Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

May 10, 2021

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Q-jet by Aerotech B6W single-use motor was tested on April 12 and May 8, 2021 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation	B6W-4,6        [single use]

Propellant	FWL

TMT Nomenclature	5 B4

Diameter	0.692”	18 mm

Overall Length	3.166”	79.15 mm

Casing Length	2.717”	69.01 mm

Loaded Mass	0.0403 lb	18.29 g

Propellant Mass	0.0062 lb*	2.8 g*

Burnout Mass	0.0278 lb	12.59 g

* = data provided by manufacturer

Burn time	1.169 ± 0.068 sec

Total Impulse	1.110 ± .053 lbf.s	4.865 ± .210 N.s

Max Impulse	2.052 ± .188 lbf	9.127 ± .838 N

Average Impulse	.948 ± .088 lbf	4.222 ± .388 N

Number of Motors Tested	10

Nominal	Measured

Delay Time Data	4 s	4.412, 4.570, 4.484 s

6 s	6.034, 6.226, 6.009, 5.692 5.435, 6.424, 6.106 s

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing