Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27516

(919) 357 5681 (c)

November 3, 2020

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech L1940X-P reload kit was tested on May 22, 2021 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation	L1940X-P        [for the 75/3840 hardware]

Propellant	Propellant X

TMT Nomenclature	4329 L1939

Diameter	2.964”	75.29 mm

Overall Length	22.063”	560.39 mm

Outside of Motor Tube	1.688”	42.86 mm

Loaded Mass	8.5 lb	3855.5 g

Propellant Mass	4.02 lb*	1825 g*

Burnout Mass	4.216 lb	1912.4 g

* = data provided by manufacturer

Burn time	2.231 ± .031 sec

Total Impulse	973.24 lbf.s	4329.2 N.s

Max Impulse	521.22 lbf	2318.5 N

Average Impulse	435.97 lbf	1939.3 N

Number of Motors Tested	3

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing