Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

October 18, 2021

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech I40N-P DMS motor was tested on October 16, 2021 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Aerotech Division, RCS Rocket Motor Components

Manufacturer’s Designation	I40N-P      DMS [end burner]

Propellant	Warp Nine

TMT Nomenclature	375 I38

Diameter	1.51”	38.25 mm

Overall Length	7.99”	202.97 mm

No thrust ring on tested version

Loaded Mass	0.793 lb	359.87 g

Propellant Mass	0.434 lb*	197 g*

Burnout Mass	0.330 lb	149.82 g

* data provided by manufacturer

Burn time	9.951 ± 0.404 sec

Total Impulse	84.28 ± 2.43 lbf.s	374.90 ± 10.82 N.s

Max Impulse	22.85 ± 0.53 lbf	101.66 ± 2.34 N

Average Impulse	8.48 ± 0.54 lbf	37.73 ± 2.422.06 N

Number of Motors Tested	3

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing