Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

October 18, 2021

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech N1975W-PS motor was tested on October 16, 2021 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Aerotech Division, RCS Rocket Motor Components

Manufacturer’s Designation	N1975W-PS                     [DMS, single use]

Propellant	White Lightning

TMT Nomenclature	13,893 N1971

Diameter	3.874”	98.39 mm

Overall Length	43.706”	1110.13 mm

Length Inside	40.956”	1040.28 mm

Length Outside	2.750”	69.85 mm

Loaded Mass	26.915 lb	12,208.43 g

Propellant Mass	16.923 lb*	7676 g*

Burnout Mass	9.040 lb	4100.47 g

* data provided by manufacturer

Burn time	7.048 ± 0.127 sec

Total Impulse	3123.29 ± 45.46 lbf.s	13,893.1 ± 202.23 N.s

Max Impulse	652.67 ± 70.75 lbf	2903.25 ± 314.73 N

Average Impulse	443.4 ± 1.55 lbf	1970.7 ± 6.93 N

Number of Motors Tested	2

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing