Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

June 3, 2002

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Q-jet by Aerotech E35W single-use motor was tested on April 29, 2022 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation	{E35-5,8,11W }       [single use]

Propellant	White Lightning

TMT Nomenclature	39 E34

Diameter	0.934”	23.73 mm

Overall Length	4.440”	112.77 mm

Outside the MMT	0.191”	4.85 mm

Loaded Mass	0.121 lb	55.09 g

Propellant Mass	0.056 lb*	25.4 g*

Burnout Mass	0.054 lb	24.54 g

* = data provided by manufacturer

Burn time	1.079 ± 0.056 sec

Total Impulse	8.853 ± 0.126 lbf.s	39.38 ± 0.56 N.s

Max Impulse	9.882 ± 0.997 lbf	43.96 ± 4.43 N

Average Impulse	7.789 ± 0.284 lbf	33.81 ± 3.11 N

Number of Motors Tested	11

Nominal	Measured

Delay Time Data	5 s	4.956, 4.431, 4.694 s

8 s	7.207, 7.157, 6.434, 6.856 s

11 s	10.720, 9.931, 9.081, 9.046, 9.558 s

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing