Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

June 3, 2002

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Q-jet by Aerotech F41W single-use motor was tested on April 29, 2022 and June 2, 2022 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation	{F41-5,8,11W }       [single use]

Propellant	White Lightning

TMT Nomenclature	45 F40

Diameter	0.935”	23.76 mm

Overall Length	4.498”	114.25 mm

Outside the MMT	0.191”	4.85 mm

Loaded Mass	0.134 lb	60.70 g

Propellant Mass	0.0661 lb*	30.0 g*

Burnout Mass	0.057 lb	25.87 g

* = data provided by manufacturer

Burn time	1.118 ± 0.251 sec

Total Impulse	10.205 ± 2.287 lbf.s	45.40 ± 10.17 N.s

Max Impulse	11.219 ± 2.626 lbf	49.94 ± 11.68 N

Average Impulse	9.054 ± 2.071 lbf	40.26 ± 9.21 N

Number of Motors Tested	14

Nominal	Measured

Delay Time Data	5 s	4.586,4.849, 5.044, 4.909 s

8 s	7.592, 6.894, 6.778, 7.178 s

11 s                          9.448, 9.732, 10.028, 8.872, 9.265, 8.938 s

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing