Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

August 13, 2022

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech K1800ST-P motor was tested on August 13, 2022 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Aerotech Division, RCS Rocket Motor Components

Manufacturer’s Designation	K1800ST-P  S                   [75/2560 hardware]

Propellant	Super Thunder

TMT Nomenclature	2443 K1776

Diameter	2.960”	75.18 mm

Overall Length	15.828”	402.0 mm

Length Inside	15.062”	383.6 mm

Length Outside	0.766”	19.45 mm

Loaded Mass	6.11 lb	2771.5 g

Propellant Mass	2.454 lb*	1113 g*

Burnout Mass	3.494 lb	1584.6 g

* data provided by manufacturer

Burn time	1.374 ± .011 sec

Total Impulse	549.36 ± 3.136 lbf.s	2443.7 ± 13.96 N.s

Max Impulse	468.13 ± 1.868 lbf	2082.4 ± 8.29 N

Average Impulse	399.37 ± 4.97 lbf	1776.5 ± 22.13 N

Number of Motors Tested	3

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing