Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

January 8, 2023

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech M2225WS-PS reload kit was tested on January 7, 2023 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation	M2225WS-PS       [For the 75/5120 case]

Propellant	Super White Lightning

TMT Nomenclature	5205 M2239

Diameter	2.958”	75.13 mm

Overall Length	27.234”	691.75 mm

Outside the MMT	1.703”	43.26 mm

Loaded Mass	10.35 lb	4694.7 g

Propellant Mass	5.139 lb*	2331 g*

Burnout Mass	4.950 lb	2245.3 g

* = data provided by manufacturer

Burn time	2.323 sec

Total Impulse	1171.2 lbf.s	5205 N.s

Max Impulse	543.68 lbf	2418.4 N

Average Impulse	503.39 lbf	2239.2 N

Number of Motors Tested	2

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing