Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

September 21, 2023

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The redesigned and reformulated Q-jet by Aerotech F41W single-use motor was tested on September 20, 2023 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation	{F41-5,8,11W }       [single use]

Propellant	White Lightning

TMT Nomenclature	48 F42

Diameter	0.937”	23.81 mm

Overall Length	4.449”	112.99 mm

Outside the MMT	0.193”	4.90 mm

Loaded Mass	0.1253 lb	56.855 g

Propellant Mass	0.060 lb*	27.1 g*

Burnout Mass	0.0541 lb	24.534 g

* = data provided by manufacturer

Burn time	1.115 ± 0.054 sec

Total Impulse	10.713 ± 0.380 lbf.s	47.651 ± 1.690 N.s

Max Impulse	12.176 ± 0.897 lbf	54.161 ± 3.990 N

Average Impulse	9.484 ± 0.673 lbf	42.185 ± 2.995 N

Number of Motors Tested	10

Nominal	Measured

Delay Time Data	5 s	5.907, 4.972, 4.904, 5.415 s

8 s	8.752, 8.341, 8.322 s

11 s               	11.004, 10.92, 11.426 s

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing