Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

April 16, 2024

Scott Kormeier

Loki Research, LLC

119 Mowrey Ln.

Theodosia, MO  65761

Dear Mr. Kormeier,

The Loki Research N5500LW reload kit was tested on March 30, 2024 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Loki Research, LLC

Manufacturer’s Designation                 N5500 LW      [made for the CTI Pro-98 6g XL hardware]

Propellant	Loki White

TMT Nomenclature	19,137 N5344

Diameter	3.868”	98.25 mm

Overall Length	54.438”	1382.7 mm

Outside the MMT	3.25”	82.55 mm

Loaded Mass	32.75 lb	14,855 g

Propellant Mass	20.243 lb*	9182 g*

Burnout Mass	12.75 lb	5783.39 g

* = data provided by manufacturer

Burn time	3.513, 3.646 sec   [avg = 3.58 sec]

Total Impulse	4219.8, 4384.4 lbf.s	18,771, 19,503 N.s

[4302.1 lbf.s]	[19,137 N.s]

Max Impulse	1487.3, 1515.2 lbf	6615.8, 6739.7 N

[1501.3 lbf]	[6677.8 N]

Average Impulse	1200.7, 1201.8 lbf	5340.8, 5346.1 N

[1201.3 lbf]	[5343.5 N]

Number of Motors Tested	2

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing