Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

April 16, 2024

Scott Kormeier

Loki Research, LLC

119 Mowrey Ln.

Theodosia, MO  65761

Dear Mr. Kormeier,

The Loki Research J-670LB reload kit was tested on March 30, 2024 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Loki Research, LLC

Manufacturer’s Designation                 	J-670 Bates   [made for the 54/1200 hardware]

Propellant	Loki Blue

TMT Nomenclature	1270 J670

Diameter	2.126”	54.00 mm

Overall Length	14.031”	1382.7 mm

Outside the MMT	0.39”	82.55 mm

Loaded Mass	2.654 lb	1204.16 g

Propellant Mass	1.274 lb*	578 g*

Burnout Mass	1.511 lb	685.19 g

* = data provided by manufacturer

Burn time	1.891 ± 0.021 sec

Total Impulse	285.47 ± 0.69 lbf.s	1269.8 ± 2.7 N.s

Max Impulse	185.28 ± 11.74 lbf	824.15 ± 52.22 N

Average Impulse	150.60 ± 2.1 lbf	669.88 ± 9.33 N

Number of Motors Tested	3

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing