Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

April 17, 2024

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech M2700W-PS reload kit was tested on March 30, 2024 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation                 	M2700W-PS     [for the 75/10240 hardware]

Propellant	White Lightning

TMT Nomenclature	10,322 N2717

Diameter	2.968”	75.39 mm

Overall Length	48.52”	1232.5 mm

Outside the MMT	1.60”	40.69 mm

Loaded Mass	19.97 lb	9058.2 g

Propellant Mass	11.629 lb*	5275 g*

Burnout Mass	7.58 lb	3438.2 g

* = data provided by manufacturer

Burn time	3.962, 3.635 sec  [average = 3.799 sec]

Total Impulse	2399.4, 2241.3 lbf.s	10,637, 9969.7 N.s

[2320.4 lbf.s]	[10,322 N.s]

Max Impulse	1248.5, 830.82 lbf	5553.5, 3695.7 N

[1039.7 lbf]	[4624.6 N]

Average Impulse	605.31, 616.25 lbf	2692.6, 2741.2 N

[610.8 lbf]	[2716.9 N]

Number of Motors Tested	2

Nominal	Measured

Delay Time Data	n.a.	n.a.

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing