Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

April 17, 2024

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Q-jet by Aerotech B14T single-use motor was tested on April 16, 2024 and complies with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	RCS Rocket Motor Components, Inc.

Manufacturer’s Designation	{B14-3,5,7T }       [18mm single use]

Propellant	Blue Thunder

TMT Nomenclature	4.97 B13

Diameter	0.693”	17.6 mm

Overall Length	3.120”	79.24 mm

Outside the MMT	0.40”	10.15 mm

Loaded Mass	0.0356 lb	16.138 g

Propellant Mass	0.0051 lb*	2.3 g*

Burnout Mass	0.0259 lb	11.736 g

* = data provided by manufacturer

Burn time	0.383 ± 0.022 sec

Total Impulse	1.119 ± 0.038 lbf.s	4.975 ± 0.169 N.s

Max Impulse	3.967 ± 0.337 lbf	17.646 ± 1.496 N

Average Impulse	2.921 ± 0.249 lbf	12.995 ± 1.108 N

Number of Motors Tested	12

Nominal	Measured

Delay Time Data	3 s	2.87, 3.652, 2.847, 3.565 s

5 s	4.81, 5.772, 4.834, 4.992 s

7 s               	6.354, 6.057, 6.459, 5.911 s

Casing Temp. [NFPA 1125 7.4.1]				Passed

B14

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing