Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

May 6, 2018

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech J435WS-13A DMS motor was tested on April 21, 2017, September 16, 2017, and May 3, 2018 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Aerotech Division, RCS Rocket Motor Components

Manufacturer’s Designation	J435WS-13A      DMS 38mm

Propellant	Super White Lightning

TMT Nomenclature	743 J442

Diameter	-	38 mm

Overall Length	14.406”	365.9 mm

Length Inside	14.035”	356.5 mm

Length Outside	0.370”	9.4 mm

Loaded Mass	1.359 lb	616.4 g

Propellant Mass	0.776 lb*	352 g*

Burnout Mass	0.532 lb	241.25 g

* data provided by manufacturer

Burn time	1.635 ± 0.094 sec

Total Impulse	166.98 ± 6.29 lbf.s	742.77 ± 27.97 N.s

Max Impulse	160.25 ± 36.63 lbf	712.85 ± 162.96 N

Average Impulse	99.28 ± 6.78 lbf	441.61 ± 30.14 N

Number of Motors Tested	13

Nominal                      Measured

Delay Time Data	6                                  7.217, 7.406, 7.085, 6.477 sec

10                                10.415, 10.655, 10.047, 10.984 s

14                                11.777, 13.333, 12.22,                                                                                                                     12.989,12.531 sec

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing