Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

August 20, 2018

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Aerotech J520W-13A RMS motor was tested on April 21, 2017, May 3, 2018, and August 14, 2018 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Aerotech Division, RCS Rocket Motor Components

Manufacturer’s Designation	J520W-13A      RMS for the 38/1080 hardware

Propellant	White Lightning

TMT Nomenclature	671 J487

Diameter	-	38 mm

Overall Length	19.77”	502.12 mm

Length Inside	19.33”	490.93 mm

Length Outside	0.433”	11.19 mm

Loaded Mass	1.670 lb	757.42 g

Propellant Mass	0.840 lb*	381 g*

Burnout Mass	0.771 lb	349.65 g

* data provided by manufacturer

Burn time	1.377 ± 0.026 sec

Total Impulse	150.9 ± 4.97 lbf.s	671.26 ± 22.09 N.s

Max Impulse	202.93 ± 23.49 lbf	902.66 ± 104.5 N

Average Impulse	109.57 ± 4.63 lbf	487.41 ± 20.60 N

Number of Motors Tested	6

Nominal                      Measured

Delay Time Data	5                                  6.227, 6.639 sec

9                                  9.332, 10.204 sec

13                                10.486, 10.047 sec                                                                                                                     12.989,12.531 sec

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing