Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

February 13, 2018

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Quest Q-Jet B4-4,6FJ single-use motor was tested on February 10, 2018 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Quest Division, RCS Rocket Motor Components

Manufacturer’s Designation	B4-4,6FJ          [18mm single use]

Propellant	Black Max

TMT Nomenclature	4.9 B4.4

Diameter	-	18 mm

Overall Length	3.17”	80.7 mm

Case Length	2.73”	69.4 mm

Loaded Mass	.0436 lb	19.77 g

Propellant Mass	.0051 lb*	2.3 g*

Burnout Mass	.0295 lb	13.39 g

Burn time	1.097 ± .030 sec

Total Impulse	1.091 ± .027 lbf.s	4.855 ± .120 N.s

Max Impulse	1.677 ± .098 lbf	7.458 ± .435 N

Average Impulse	0.990 ± .033 lbf	4.405 ± .149 N

Number of Motors Tested	12

Nominal	Measured

Delay Time Data             4 s	3.943, 3.812, 4.042, 3.757, 3.301, 3.81 sec

6 s			6.307, 6.026, 6.173, 6.753, 6.309, 6.025 sec

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing