Tripoli Motor Testing Division

223 NC Highway 54 West

Chapel Hill, NC 27514

(919) 357 5681 (c)

May 12, 2018

Gary Rosenfield

RCS Rocket Motor Components, Inc.

2113 West 850 North Street

Cedar City, UT   84721

Dear Mr. Rosenfield,

The Quest Q-Jet D16-4,6,8FJ single-use motor was tested on May 2, 2018 and is in compliance with NFPA 1125.  The motor is certified for hobby rocketry use by the members of the Tripoli Rocketry Association and all approving organizations with reciprocal certifying agreements.

Motor Manufacturer	Quest Division, RCS Rocket Motor Components

Manufacturer’s Designation	D16-4,6,8FJ          [18mm single use]

Propellant	Black Max

TMT Nomenclature	12 D16

Diameter	-	18 mm

Overall Length	3.137”	79.7 mm

Case Length	2.706”	68.7 mm

Loaded Mass	.0565 lb	25.6 g

Propellant Mass	.0276 lb*	12.5 g*

Burnout Mass	.0290 lb	13.1 g

Burn time	0.792 ± .025 sec

Total Impulse	2.781 ± .069 lbf.s	12.37 ± .31 N.s

Max Impulse	5.103 ± .221 lbf	22.3 ± 0.98 N

Average Impulse	3.506 ± .158 lbf	15.60 ± .70 N

Number of Motors Tested	11

Nominal	Measured

Delay Time Data             4 s	4.01, 4.219, 4.764 sec

6 s			6.455, 6.644, 6.354, 5.663 sec

8 s			7.274, 9.012, 6.945, 7.891 sec

Casing Temp. [NFPA 1125 7.4.1]				Passed

Sincerely,

Alan C. Whitmore

Chair, Tripoli Motor Testing